# The “BULL” & “The Facts”

## Carrying Case

Dimensions			22” x 15” x 9.5”

Weight			Approx.  8 pounds

## Overall TI Unit

## Weight

W/ Battery		5.75 pounds

W/out Battery	5.00 pounds

Dimensions 			Height : 12 “            Length : 10 “            Width : 6 ”

Heat Test  			650F  (343C)  for 5 minutes

Operating Range		-40F  (-40C)   to   650F  (343C)

#### Impact / Drop Test		Dropped 3 consecutive times from 3 feet w/ no noted

physical or operational damage

## Casing

Shell Material		Ultem Thermoplastic

### Sealing			Silicone

Strap Material		Nomex & Leather

### Display Cover		Polycarbonate

Heat Test			650F  (343C)  for 5 minutes

Impact / Drop Test		Dropped 3 consecutive times from 3 feet w/ no noted

physical or operational damage

Core / Detector

Type				Uncooled Ceramic Focal Plane Array Detector

Resolution			320 x 240 array

Sensing Material		Barium Strontium Titanate Ceramic (BST)

Spectral Response		8 – 14 	Microns

Thermal Stabilization	Thermoelectric Cooler

Update Rate			30 Hz

Temperature Sensitivity	0.05 C

Video Output			RS-170 Signal (1.4 volts P-P)

Lens

Material			Germanium

Lens Size			18 mm

### Field of View			59 (diagonal)

Distances		Optimal Focus = 3 feet (1 meter) to infinity

Electrical System

Power Source			NiCad or NiMH  Rechargeable Battery

Output			10V

Capacity			1100 mA.hr

Operating Time		1 – 1.5 hours

Charger			Single Battery, 120 VAC or 12 VDC

Switch Cycle Test		1,000,000 cycles

Battery Life			1,500 charge cycles

Battery Weight		0.75 pounds

Display

Type				Liquid Crystal Display (LCD)

Size				4”

Dot Pitch			.259mm (V) x .210mm (H)

Dot Format			234 X 380 Dots

Pixels				89,622

Pixel Configuration		R-B-G Delta Configuration

Display Method		TFT Active Matrix

Input Signal Level		1.0V P-P (Positive) 75 Ohm

Back Light			CCFT Backlight

Transmitter

Mounting			Internal

Signal Type			Analog

Antenna Type		Dipole

### Transmitting Frequency	2.4 GHz

Power Output		400 mW

Power Supply			Internal,

Power Consumption		2.8 W

Frequency Selection		2 Channel Switch  (2.458 or 2.474 GHz)

FCC License			Part 90

## Receiver

Dimensions

Box			Length : 4.5 “          Width : 4.5 “          Height : 1.5 “

Patch Antenna	Length : 13 “           Width : 13 “           Height : 1.5 “

Power Requirements		11-14.5 VCD (Center Positive)

### Power Consumption		250 mA

Power Input			Coaxial 2.1mm x 5.5 mm Center- Positive Connector

Power Supply			120 VAC or 12 VDC

Operating Temperature	14 F to 122 F

Signal Type			Analog

Receiving Frequency		2.458 GHz or 2.474 GHz

Video Outputs		Two (2) 75-ohm 1-volt  P-P  video outputs (female BNC)

Range

Unobstructed Line-of-Sight	 10 miles

Obstructed Line-of-Sight	 400 feet